# A Predictive-Text Interface for Brain-Computer Interfaces

Literature Review, Requirements and Implementation Plan

## Literature Review

The literature themed around predictive-text BCIs is structured across 4 related domains: (1.1) The neurophysiological substrate & physical bandwidth constraints of non-invasive spellers, (1.2) the integration of language models into BCI decoders, (1.3) human-computer interaction (HCI) trade-offs in assistive text entry, and (1.4) clinical benchmarks centred around subjects and information metrics.

### 1.1 The Neurophysiological Substrate & Physical Bandwidth Bottlenecks

#### Farwell & Donchin (1988) – Toward a mental prosthesis utilising ERPs

Farwell and Donchin investigated whether the P300 ERP could serve as a way of allowing communication without motor needs. They designed a 6x6 matrix of letters and system commands where rows and columns flashed randomly for 100ms within an oddball paradigm to elicit P300 ways at the intersection of the relevant row and column. 4 healthy volunteers between the age of 20 to 36 first participated in a session to successfully spell ‘BRAIN’ to drive a speech synthesizer. The second session evaluated 125ms and 500ms ISIs across 10 blocks of 30 trials using bootstrap sampling across 4 detection algorithms, namely: SWDA, Peak Picking, Area Summation and Covariance. The authors optimised each subject’s algorithm and ISI post-hoc (which differed between each subject) to achieve a result of 95% accuracy with a mean speed of 26s per character with a backspace correction having over 99.9% at 0.18 bits/s. The study suffers from various constraints that were imposed. EEG recording occurred at only a single electrode, Pz, leaving the system with a high risk to noise that would require filtering. The volunteers are not from a wide range of backgrounds that could represent the general population, and the study did not integrate language constraints to statistically predict the next letter and reduce the search space – leaving the system to be impractical and exhausting for the participant.

#### Wolpaw (2007) – BCIs for communication and control

Wolpaw establishes mutual adaptation, by both user and system as central to success. The subject learns to regulate their neural output, and the system uses decoding algorithms that adapt to the user’s real-time signals. To simulate practical clinical reality, the researcher deployed a home BCI operating on the BCI2000 platform, featuring a simple 16 channel electrode cap and a sequential menu-based format that is configurable for the user’s needs. A locked-in ALS patient that used the system independently for 6 to 8 hours daily to write emails evaluated it as superior to eye-gaze tracking.

#### Rezeika et al. (2018) – BCI spellers: A review

A comprehensive systematic review analysing 69 BCI speller studies and their shared qualities. It classed non-invasive spellers into the following categories:

- P300, 65% present

- SSVEP, 23% present

- Motor Imagery, 6% present

- Hybrid, 6% present

It found that only 14.5% of studies taken into account integrated word prediction, with the vast majority using small, non-disabled participant cohorts in a noise-free laboratory environment with inconsistent metrics reported. This shows the area of predictive language modelling with BCIs is under-researched, a gap that I’ll address.

### 1.2 Integrating Language Models into Non-Invasive BCI Decoding

#### Ryan et al. (2011) – Predictive Spelling With a P300-Based BCI: Increasing Rate of Communication

The authors conducted the first controlled online comparison between a conventional non-predictive speller (NS) and a predictive speller (PS), using the external word-prediction program Quillsoft WordQ2. 24 healthy participants used an 8x9 matrix to copy and spell letters and correct errors for a 58-character sentence under counterbalanced quasi-random conditions. They had to select digits 1-7 to choose suggested words in an adjacent window, with the PS having a higher overall task completion time of 12.43min than the 20.20min achieved by the NS, and higher output rate of 5.28 characters/min compared to 3.76 characters/min. The study used a moving average of repetitions to reduce the effect of noise, it was found that the PS has a lower accuracy of 84.88% against 89.80%, and that the P300 amplitudes were attenuated due to what the authors deem to be from the increase in cognitive workload and having to shift gaze between matrix and prediction window. Additionally, misclassifying a word suggestion resulted in a harsh penalty of multiple backspaces required. The lower accuracy is found to be justifiable for subjects, where speed takes priority.

#### Kaufmann et al. (2012) – A user-centred BCI Including Auto-Calibration and Predictive Text Entry

This builds upon the previous paper, by resolving the divided attention issue that occurred previously, by presenting up to 6 predictions in the matrix. The study evaluated 20 healthy volunteers in an online sentence-copying task with an automated classifier calibrator, reducing the time to spell a 36-character sentence by 42% without a significant decrease in accuracy. Through average ERP analysis, the study confirmed that P300 amplitudes were not attenuated under the prediction text entry as compared to the standard character-by-character spelling. A limitation of the study is that word completions were derived from an unweighted word frequency list rather than context-aware models such as n-grams, meaning prior words were not considered for the prediction.

#### Speier et al. (2012) – NLP with Dynamic Classification

The study introduced natural language processing into the P300 signal classification pipeline by combining SWDA with a Naïve Bayes classifier and a character trigram language model trained on Brown Corpus. It was tested offline on 6 subjects across 9 5-letter target words, using language model priors with dynamic stopping, continuing flash sequences until posterior confidence exceeds a threshold of 0.95. The trigram model increased character classification accuracy from 82.97% to 92.59% and ITR from 22.06 to 33.15 bits/min. Unfortunately, the model was trained on a set of data that is not domain specific and does not complete words.

#### Parthasarthy et al. (2026) – Multi-Model Analysis with Performance Bounds

The authors established the empirical performance bounds of integrating modern autoregressive LLMs into P300 spellers using EEG data from 78 healthy subjects who typed 7892 characters, comparing models from GPT-2 to LLaMA 3.3. The best model, GPT-J, achieved an ITR of 75.36 bits/min. This is within 5% of the theoretical word-level oracle bound of 78.21 bits/min, and 73% of the physiological timing ceiling of 103 bits/min. Without calibrating the model to each individual subject, it still reaches an ITR of 59.90 bits/min. Making the LLM did not continue to improve performance after moderate size.

### 1.3 Human-Computer Interaction (HCI) & Text-Entry Trade Off

#### Trnka et al. (2008) - Word Prediction & Communication Rate in AAC

The authors examined the relationship between word prediction algorithms, cognitive processing overhead, and net communication rate in AAC by testing 28 adult participants on an on-screen keyboard configured with a 1.5s keypress delay to simulate motor impaired typing. Comparing letter-by-letter typing, a frequency list, and a trigram model with backoff showed that the trigram increased net communication rate by 58.6% over letter-by-letter typing and 45.4% over basic prediction. This includes a 53.6% keystrokes saving, 93.6% word prediction utilisation, and subjects reporting feeling less fatigue than with other methods.

#### Quinn & Zhai (2016) – Cost-Benefit Study of Text Entry Suggestions Interaction

The study investigated the cost-benefit trade-off of interface assertiveness in suggesting predictions across 17 touch-typists. There were 3 models taken into account:

- Introverted, no word predictions/suggestions given, 3.09 char/s typing speed

- Extraverted, 3 word suggestions always shown, 2.81 char/s typing speed

- Ambiverted, suggestions only shown when crossed a confidence threshold, 2.66 char/s typing speed

Visual search and time to verify suggestions outweighed keystroke savings, however it should be noted that the context behind the study is that the medium is for a mobile device – where keypress costs is significantly lower than that with BCIs.

#### Cai et al. (2024) – Abbreviation Expansion

The authors developed SpeakFaster, an AAC interface that uses LLMs to understand highly abbreviated keystrokes. For instance, with Google’s C-PAK, ‘ishpitb’ produces ‘I saw him playing in the backyard’. If the suggestion is wrong, the user can correct it with 2 fallback features, KeywordAE which lets them add a keyword and FillMask which replaces an incorrect word. In offline tests, SpeakFaster needed 57% fewer motor actions than a traditional predictive keyboard, and 2 subjects with ALS who used eye gaze demonstrated an increase of typing speed by 29% to 60%. Although the application differs from my use case, the core technology is of relevance.

### 1.4 User-Centred Clinical Benchmarks & Evaluation Metrics

#### Huggings, Wren, & Gruis (2011) – Opinions & Priorities of Potential Users with ALS

The study surveyed 61 individuals with ALS to establish the expectations from target users. It found that users expect >=90% accuracy, a typing speed of >=15-19 letters/min, and a standby false positive rate under one accidental selection per 2-4 hours. Therefore, there is reason to believe that there exists real-world demand for assisted technology to meet these requirements, that unassisted P300 spellers cannot meet as we have observed in other papers in my literature review.

#### Speier, Arnold & Pouratian (2013) – Evaluating True BCI Communication Rate

The study demonstrated that ITR is invalid for BCIs for our use case of word prediction. ITR assumes, falsely, that characters have an equal probability of occurring, memoryless independence, and random errors. This means ITR rewards fast and inaccurate typing. Instead, the study suggests using MI that uses n-gram priors and systematic error matrices and upon reviewing 11 BCI studies, found that 6 of these reversed performance rankings and altered the optimal parameters.

## 2. Literature Synthesis & Gap Analysis

After reviewing the literature, there are numerous findings that define the scope and technical direction of the dissertation.

- Non-invasive EEG-based spellers are constrained by how quickly the brain can respond to stimuli, and how many responses the system requires to quantify a selection accurately. Without predictive spelling, P300 spellers achieve 2-5 characters per minute which we know is well below the expectations of ALS patients.

- The benchmarks show that existing language models already perform close to the theoretical best-case performance for word-level prediction, within 5%. Moving from smaller to larger models shows an improvement in bit rate by 1-2%, which does not justify the increase in response time associated with it. Hence, interface design and interaction efficiency is a more effective approach to pursue than relying on model size.

- With the limited studies that do integrate predictive language modelling, having an interface that does not split the subject’s attention as much as otherwise, and filtering with confidence intervals show that combining both approaches could be promising.

Therefore, the project aims to address this gap by developing an in-matrix, confidence-gated predictive-text BCI interface. By combining local n-gram/transformer language priors with dynamic stopping and commands that allow swift error recovery, the system aims to maximise keystroke savings whilst minimizing visual search overhead in an effort for the system to be viable in home settings for the daily needs of people.

## 3. System  Requirements Specification

To bridge the gap between language modelling and clinical usability, this dissertation introduces 3 novel research contributions that it wishes to explore.

### 3.1 Novel Problem Formulations

#### Novelty 1: Confidence-Gated Interface Assertiveness

Instead of always displaying predictions, the interface adapts its assertiveness to decoder confidence and language-model certainty across distinct operational states.

- Low confidence (Introverted) – Shows only letters, avoids distracting the user with weak or ambiguous predictions when certainty is low, eliminates visual search overhead.

- Medium confidence (Ambiverted) – Shows 3 likely word completions inside the matrix. They are located inside the matrix cells, allowing for subject to inspect and select words without gaze-shifting.

- High confidence (Extraverted) – Promote the best completion to a large, low-cost ‘accept’ targe, or reducing flashing for unlikely options. This allows for rapid single-trial word execution when both EEG and LM converge strongly.

This directly is an extension of Quinn and Zhai’s suggested idea of assertiveness into a BCI setting, where the cost of inspecting a wrong suggestion is much higher than that on a phone keyboard.

#### Novelty 2: Error-Aware Prediction Selection (Expected Cost Minimisation)

Most predictive spellers choose the highest probability next word by unweighted maximum likelihood. In BCIs, however, choosing an incorrect word prediction introduces a substantial time loss, forcing the user to endure multiple consecutive backspace trials (unless a clear command exists). Instead, this dissertation will choose a prediction that minimises total cost. For example, a highly likely but long word may be worse than a slightly less likely short word if rejecting it means many backspaces and re-spelling.

#### Novelty 3: Adaptive Flashing Driven By Language Priors

Rather than flashing rows and columns uniformly in a pseudo-random fashion, this contribution uses the language model to deprioritise implausible candidates whose prior probability is near 0 given preceding context.

### 3.2 System Requirements Specifications

| ID | Priority | Requirement Description | Category | Justification |
| --- | --- | --- | --- | --- |
| 01 | MUST-HAVE | In-Matrix Visual Layout, with 3 prediction cells within | Functional (UI) | Eliminates divided visual attention, preventing P300 amplitude attenuation. |
| 02 | MUST-HAVE | Contextual LM Prior Engine | Functional (Algorithm) | Biases classifier likelihoods towards linguistically probable tokens. |
| 03 | MUST-HAVE | Backspace & Undo Command | Functional (UI) | Limits time penalty for errors. |
| 04 | MUST-HAVE | Evaluation Metrics KS, OCM, MI | Evaluation | For successful evaluation of non-equiprobable system |
| 05 | SHOULD-HAVE | Confidence gated assertiveness | Novelty (UI) | Prevents visual search overhead |
| 06 | SHOULD-HAVE | Error aware cost sensitive selection | Novelty (Algorithm) | Penalises long, high-risk words |
| 07 | SHOULD-HAVE | Uncertainty Aware Adaptive Flashing | Novelty (Stimulus) | Minimises unnecessary stimulus repetitions whilst preserving P300 assumptions |
| 08 | COULD-HAVE | Abbreviation Expansion | Functional | SpeakFaster |

## 4. Project Plan & Work Breakdown Structure

| Phase | Duration | Key Work | Target Milestone |
| --- | --- | --- | --- |
| Phase 1: Design and Implement UI (Non-Interactive) | Weeks 1-2 | Space for predictions, letters and commands, shape of matrix decided. Basic Intensifications working. | M1 – Validated Non-Interactive UI |
| Phase 2: Algorithmic Foundations | Weeks 3-7 | Implement prefix tree dictionary and n-gram conditional probability engine. Implement the expected cost minimisation algorithm. Implement confidence assertiveness. | M2 – Validate Cost Aware Language Inference Engine |
| Phase 3: EEG | Weeks 8-12 | Record and decode EEG streams from BCI | M3 – Validate EEG stream |
| Phase 4: Interactive UI | Weeks 13-18 | Establish Bayesian posterior engine that combines EEG likelihoods and LM priors. Implement. Implement uncertainty-aware intensifications. Make UI interactive, i.e. able to spell and use commands | M4 – Validate Interactive UI |
| Phase 5: Simulation & Benchmarking | Weeks 19-22 | Construct automated transcription | M5 – Conduct experiment with subjects and gather results |
| Phase 6: Synthesis | Weeks 23 – 25 | Finalise report | M6 – Complete report in time for presentation |

## 5. Glossary

EEG – Electroencephalography, a non-invasive neuroimaging technique that records spontaneous electrical activity of cortical neuronal ensembles via conductive electrodes placed on scalp.

ERP – Event-Related Potential, a stereotyped electrophysiological voltage fluctuation time-locked to a specific sensory, cognitive or motor stimulus, extracted from the ongoing EEG via epoch averaging.

P300 – An endogenous, positive ERP deflection occurring 200-400ms after stimulus onset, elicited over centro-parietal scalp regions when a user detects a rare, task-relevant target.

Oddball Paradigm – An experimental visual, auditory, or tactile presentation protocol in which rare, task-relevant target stimuli are pseudo-randomly shown among frequent non-target stimuli to evoke a P300 response.

ISI – Inter-Stimulus Interval, a temporary delay between onset of one stimulus intensification and the next within a trial.

ITI – Inter-Trial Intervals, the total time elapsed between the completion of one trial and onset of the next.

SNR – Signal-to-Noise Ratio, the ratio of target ERP amplitude to background electrical and spontaneous physiological noise, this is typically enhanced by repeated trials of stimulus intensification and spatial filtering.

SMR – Sensorimotor Rhythms, oscillatory EEG activity recorded over the sensorimotor cortex, typically alpha and beta waves, that desynchronise during motor execution or motor imagery.

SSVEP – Steady-State Visual Evoked Potential, a periodic brain response elicited in the visual cortex when focusing upon a visual stimulus flickering at a constant frequency.

SWDA – Stepwise Discriminant Analysis, a supervised classification technique that iteratively adds and removes discriminant time-domain EEG features to distinguish target from non-target epochs.

ALS – Amyotrophic Lateral Sclerosis, also known as ‘Locked-In Syndrome’

Auto-Calibration – Adaptive machine learning algorithms that configure classifier weights in the background from unsupervised usage, removing need for manual calibration sessions.

Dynamic Stopping – An online decoding strategy that evaluates classifier confidence after each stimulus repetition and terminates intensification once a confidence threshold has been reached.

ASCV – Across-Subject Cross Validation, decoders are trained from data from historical subjects and tested on an unseen user to evaluate calibration effectiveness.

WSCV – Within-Subject Cross Validation, decoders are trained and tested on partitions of data from the same subject.

BCI2000 – An open-source software platform used for real-time BCI data acquisition, stimulus presentation, and signal processing.

Flashboard – A visual display grid containing alphanumeric characters, symbols, word predictions and commands that undergo intensifications in an ERP speller.

AAC – Augmentative and Alternative Communication, the domain of supplementing or replacing natural speech and writing for individuals with severe communication impairments.

ALS – Amyotrophic Lateral Sclerosis, a progressive neurodegenerative disease that leads to degeneration of upper and lower motor neurons, leading to severe paralysis, loss of speech and respiratory failure.

LIS – Locked-In Syndrome, a medical condition where the subject is in a complete paralysis of voluntary muscles with preserved consciousness and cognitive function.

Language Model Prior – The conditional probability of a character or word by a statistical or neural language model, multiplied by EEG classifier likelihoods to bias decoding towards linguistically probable sequences.

N-gram Model – A statistical language model that computes the probability of a token based on the occurrence frequency of the preceding tokens, for instance trigrams.

OOV – Out-of-Vocabulary, words entered by the user that do not exist within the language model’s training corpus, requiring fallback of either character-by-character spelling or smoothing algorithms such as Kneser-Ney.

KS/KSR – Keystroke Savings, a AAC metric calculating the percentage reduction in physical keypresses or neural selections achieved by predictive text relative to spelling each word character by character.

OCM – Output Characters per Minute, a text entry speed metric measuring the net rate of correct characters produced in the final message, divided by the total time elapsed. This includes corrective actions such as backspace.

ITR – Information Transfer Rate, Shannon channel capacity combining selection speed, accuracy, and the number of targets. It assumes memoryless and equiprobable targets, and random errors.

MI – Mutual Information Rate, an evaluation metric that replaces ITR’s equiprobable assumptions with uniform natural language prior probabilities and empirical error matrices.

Interface Assertiveness – The frequency at which an interactive user interface displays predictive suggestions.

Inspection Overhead – The cognitive and perceptual delay required for a subject to shift visual focus, read, verify and evaluate whether to accept or reject predictive suggestions.

Abbreviation Expansion – An AAC text-entry method where subjects enter abbreviations that a language model expands into complete, grammatically fluent phrases.

Oracle Bound – A best case benchmark, the best performance attainable within a fixed decoding architecture when the language-model prior is replaced by a perfect next-token predictor.

## 6. References

[1] L. A. Farwell and E. Donchin, "Talking off the top of your head: Toward a mental prosthesis utilizing event-related brain potentials," Electroencephalogr. Clin. Neurophysiol., vol. 70, no. 6, pp. 510–523, Dec. 1988, doi: 10.1016/0013-4694(88)90149-6. Available: https://doi.org/10.1016/0013-4694(88)90149-6

[2] S. Cai et al., "Using large language models to accelerate communication for eye gaze typing users with ALS," Nat. Commun., vol. 15, no. 1, p. 9449, Nov. 2024, doi: 10.1038/s41467-024-53873-3. Available: https://doi.org/10.1038/s41467-024-53873-3

[3] J. R. Wolpaw, "Brain-computer interfaces (BCIs) for communication and control," in Proc. 9th Int. ACM SIGACCESS Conf. Comput. Access. (ASSETS '07), Tempe, AZ, USA, 2007, pp. 1–2, doi: 10.1145/1296843.1296845. Available: https://doi.org/10.1145/1296843.1296845

[4] D. B. Ryan, G. E. Frye, G. Townsend, D. R. Berry, S. Mesa-G, N. A. Gates, and E. W. Sellers, "Predictive spelling with a P300-based brain-computer interface: Increasing the rate of communication," Int. J. Hum.-Comput. Interact., vol. 27, no. 1, pp. 69–84, Jan. 2011, doi: 10.1080/10447318.2011.535754. Available: https://doi.org/10.1080/10447318.2011.535754

[5] T. Kaufmann, S. Völker, L. Gunesch, and A. Kübler, "Spelling is just a click away – A user-centered brain–computer interface including auto-calibration and predictive text entry," Front. Neurosci., vol. 6, p. 72, May 2012, doi: 10.3389/fnins.2012.00072. Available: https://doi.org/10.3389/fnins.2012.00072

[6] W. Speier, C. Arnold, J. Lu, R. K. Taira, and N. Pouratian, "Natural language processing with dynamic classification improves P300 speller accuracy and bit rate," J. Neural Eng., vol. 9, no. 1, p. 016004, Feb. 2012, doi: 10.1088/1741-2560/9/1/016004. Available: https://doi.org/10.1088/1741-2560/9/1/016004

[7] A. Rezeika, M. Benda, P. Stawicki, F. Gembler, A. Saboor, and I. Volosyak, "Brain–computer interface spellers: A review," Brain Sci., vol. 8, no. 4, p. 57, Mar. 2018, doi: 10.3390/brainsci8040057. Available: https://doi.org/10.3390/brainsci8040057

[8] K. Trnka, J. McCaw, D. Yarrington, K. F. McCoy, and C. Pennington, "Word prediction and communication rate in AAC," in Proc. Tele-Health and Assistive Technologies (Tele-health/AT '08), Baltimore, MD, USA, 2008, pp. 19–24. Available: https://www.scitepress.org/Papers/2008/31275/31275.pdf

[9] P. Quinn and S. Zhai, "A cost–benefit study of text entry suggestion interaction," in Proc. 2016 CHI Conf. Hum. Factors Comput. Syst. (CHI '16), San Jose, CA, USA, 2016, pp. 83–88, doi: 10.1145/2858036.2858305. Available: https://doi.org/10.1145/2858036.2858305

[10] W. Speier, C. Arnold, and N. Pouratian, "Evaluating true BCI communication rate through mutual information and language models," PLoS ONE, vol. 8, no. 10, p. e78432, Oct. 2013, doi: 10.1371/journal.pone.0078432. Available: https://doi.org/10.1371/journal.pone.0078432

[11] J. E. Huggins, P. A. Wren, and K. L. Gruis, "What would brain-computer interface users want? Opinions and priorities of potential users with amyotrophic lateral sclerosis," Amyotroph. Lateral Scler., vol. 12, no. 5, pp. 318–324, Sep. 2011, doi: 10.3109/17482968.2011.572978. Available: https://doi.org/10.3109/17482968.2011.572978

[12] N. Parthasarathy, J. Soetedjo, S. Panchavati, N. Parthasarathy, D. Lee, C. Arnold, N. Pouratian, and W. Speier, "Near-optimal P300 speller performance using large language models: A multi-model analysis with performance bounds," PLoS ONE, vol. 21, no. 10, p. e0349281, Oct. 2026, doi: 10.1371/journal.pone.0349281. Available: https://doi.org/10.1371/journal.pone.0349281